(Key in Your Company Name Here)

# Silica Exposure Control Plan for

# Table 1 Applications

For

(Key in the Job Name Here)

Jobsite Address

_

_

_

_

Overview

For the silica dust producing task(s) described below (Company Name) will use Table 1 of OSHA’s Respirable Crystalline Silica Standard to protect its workers from overexposure to respirable crystalline silica.

Purpose

The purpose of this written silica exposure control plan is to protect (Company Name) workers from overexposure to respirable crystalline silica. Overexposure can lead to series health problems such as silicosis, lung cancer, chronic bronchitis, kidney disease, and autoimmune diseases. Therefore, (Company Name) will implement all applicable parts of Table 1 to ensure that all affected workers:

Use only equipment described in Table 1;

Perform only the tasks described in Table 1;

Implement all required engineering control methods established for each applicable piece of equipment/task described in Table 1;

Implement all required work practice control methods established for each applicable piece of equipment/task described in Table 1;

Implement all required respiratory protection functions; and

Comply with the minimum Assigned Protection Factors (APF) for respiratory protection as described in Table 1.

(Name of Designated Competent Person) is (Company Name’s) designated competent person for silica related activities that affect (Company Name) workers on this project. (Name of Designated Competent Person) is responsible for implementation of this exposure control plan, and will perform frequent and regular inspections of applicable areas of the jobsite, materials, and equipment to ensure that it is being properly implemented. (His/her) responsibilities include, but are not necessarily limited to ensuring that:

Affected workers are only using the equipment/performing the silica related tasks described in Table 1;

Affected workers have received appropriate safety and health training on respirable crystalline silica generating tasks, accompanying hazards, and effective protective measures;

Appropriate Table 1 engineering controls are established and properly implemented;

Appropriate Table 1 safe work practices are established and properly implemented;

Appropriate respiratory protection is used when required. When respiratory protection is required, (Name of Designated Competent Person) will ensure that minimum Assigned Protection Factors (APF) are being carefully followed, and respirator use is in compliance with all applicable respiratory protection standards;

Housekeeping practices limit exposure to respirable crystalline silica as much as possible; and

The effectiveness of this exposure control plan is evaluated at least annually where applicable, and updated as necessary to keep affected workers from overexposure to respirable crystalline silica.

Silica Dust Producing Tasks

On this project (Company Name) will perform the following silica dust producing task(s).

(Delete all that do not apply and add any that apply, but which are not already listed from Table 1. Also, add detail such as where on the project the task(s) will be performed, how many of your workers will be directly affected, how many holes or what size cuts you’ll be making, etc.  For example… “Use of handheld impact drill by one worker in the corridor leading to the mechanical room. The worker will drill thirty 3/8” pipe hanger holes 2 ¾” deep into the poured concrete ceiling.

Handheld drill work on concrete (add details)

Handheld impact drill work on concrete (add details)

Handheld rotary hammer drill work on concrete (add details)

Stand mounted drill work on concrete (add details)

Stand mounted impact drill work on concrete (add details)

Stand mounted rotary hammer drill work on concrete (add details)

Rig mounted core drill/saw work on concrete (add details)

Handheld power saw work on concrete (add details)

Walk-behind saw work on concrete (add details)

Jackhammer work on concrete (add details)

Handheld powered chipping tool work on concrete (add details)

Engineering Controls

(Describe the engineering controls from Table 1 that correspond with the equipment/silica dust producing tasks described above. For example… “Handheld impact drills will be used with a shroud and HEPA filtered dust collection system made by -----. The HEPA filter has an efficiency rating greater than 99%. The system is equipped with a built-in filter cleaning mechanism. A HEPA filtered vacuum system will be used to clean any remaining dust out of the finished holes.)

Work Practice Controls

(Describe the work practice controls from Table 1 that correspond with the equipment/silica dust producing tasks described above. For example… “When using handheld impact drills, affected workers will operate and maintain the tool in accordance with the manufacturer’s instructions to minimize dust emissions.)

Respiratory Protection

(Describe the respiratory protection requirements from Table 1 that correspond with the equipment/silica dust producing tasks described above. For example… “When using handheld impact drills in accordance with Table 1 engineering and work practice controls, affected workers will not be required to use respiratory protection.)

- (When respiratory protection is required for Table 1 equipment/task that will be used/performed by your workers, include the following section on respiratory protection. Otherwise, delete it.)

- When respiratory protection is required all affected workers will:

Received appropriate training on respiratory protection, including the contents of OSHA’s Respiratory Protection Standard, and proper respirator selection, use, maintenance, storage;

Completed a medical evaluation and been approved to work while wearing a respirator; and

Completed fit testing procedures that have established an appropriate size, brand, and style of respirator for adequate protection from overexposure to respirable crystalline silica on this project.

Housekeeping Measures to Limit Worker Exposure

Dry sweeping, and dry brushing are not permitted by (Company Name) workers on this project.

Use of compressed air to clean clothing or surfaces in affected work areas are not permitted by (Company Name) workers on this project.

A HEPA vacuum will be used for all silica containing dust cleanup.

Affected Area Access Restrictions

(Company Name) restricts access by all others to areas where (Company Name) workers are performing (insert task(s) you selected from Silica Dust Producing Tasks above), and restricts access to its own affected workers who must perform work in areas where other trades are pulverizing silica containing building materials.

Prior to starting work on any project where respirable crystalline silica exposure is a concern (Name of Designated Competent Person) will meet with all other affected employers to determine whether (Company Name) workers could be exposed to respirable crystalline silica from the work of other trades on the project. Where potential exposures are identified (Name of Designated Competent Person) will document the operations, their locations on the project, and when they will be performed.

Prior to starting work on any project where respirable crystalline silica exposure is a concern (Name of Designated Competent Person) will meet with all (Company Name) workers to inform them about the silica exposures on the project and the necessary affected area restrictions.

When (Company Name) workers are performing tasks that Generate Respirable Crystalline Silica:

The affected work area will be barricaded with stanchions, and yellow and black caution tape.

The barricaded area will be large enough to prevent other trades in the area from overexposure to respirable crystalline silica, provided that they do not breach the barricade.

Signs stating, “Caution – Silica” will be posted around the perimeter of the barricaded areas so that other trades will know why they should not breach the barricade.

(Name of Designated Competent Person) will inform all other affected employers on the project about the silica generating tasks that will be performed by (Company Name), their locations on the project, and when they will be performed.

When (Company Name) workers must work in close proximity to other trades that are pulverizing silica containing building materials:

Affected (Company Name) workers will not enter the work area, but will report the issue to (Name of Designated Competent Person).

(Name of Designated Competent Person) will reschedule the mechanical construction work in the affected area to another time when exposure to respirable crystalline silica is not a concern.

When work in the affected area can’t be rescheduled, access to affected (Company Name) workers will not be restricted, but (Name of Designated Competent Person) will ensure that they are implementing the necessary safe work practices and protective measures to prevent overexposure to respirable crystalline silica in those work areas.

Review/Evaluation of This Silica Exposure Control Plan

(Name of Designated Competent Person) will evaluate the effectiveness of this written silica exposure control plan at least annually, and update it as necessary to keep affected workers from overexposure to respirable crystalline silica on this project.