Revision Record

| DATE | REVISION # | APPROVED BY | DESCRIIPTION |
| --- | --- | --- | --- |
| 12/9/2011 | 1 | Ed Lewis | Added section 3.1 (General Waste Management). Added 2nd paragraph in section 17.0 (Training) to include specific requirements for waste disposal. |
| 11/17/2017 | 2 | Michael S. Evans | Added bullets to section 3.1 concerning waste management |

All revisions must be approved by WW Gay Safety Officer.

TABLE OF CONTENTS

1.0	PURPOSE AND SCOPE	3

2.0	RESPONSIBILITIES	3

2.1	Project Superintendant	3

2.2	General Foreman	3

2.3	Safety Specialist	3

2.4	Employees	3

3.0	Housekeeping	4

3.1	General Waste Management	5

4.0	Personal Protective Equipment (PPE)	5

4.1	Hard hats must be worn in the following manner;	5

4.2	Hearing protection	6

4.3	Safety Glasses	6

4.4	Foot Protection	6

4.5	Task Specific PPE	6

5.0	Falls Protection	6

6.0	Confined Space	7

7.0	Scaffolds and Ladders	7

7.1	Scaffolds	7

7.2	Ladders	7

8.0	Hoisting & Lifting Equipment	7

9.0	Vehicles and Mobile Equipment	8

10.0	Tools and Equipment	8

10.1	Electrical Cords	8

10.2	Powered Hand Tools	8

10.3	Gas, Propane, & Hydraulic Powered Tools	9

10.4	Attachments	9

10.5	Hand Tools	9

11.0	FIRE Protection	10

12.0	Excavations	10

13.0	Administrative	10

14.0	Procedures	11

15.0	Hazardous chemicals	12

16.0	Completing the daily job assessment form	12

17.0	Training	12

# PURPOSE AND SCOPE

This procedure provides the guidelines for the proper inspection of all job sites that WW Gay employees are working on. This procedure also outlines the proper documentation of the Daily Job Site Inspection form.

This procedure applies to all WW Gay job sites where the duration of the job is longer than four (4) hours.

Note: Due to the nature of WW Gay’s work, some job sites may have more stringent work rules than the requirements in this procedure. WW Gay will follow the customer’s requirements only if they are more stringent than the requirements established in this procedure. This is the minimum requirements for WW Gay employees and subcontractors.

# RESPONSIBILITIES

## Project Superintendant

- The Project Superintendant is responsible for;

- Implementing and enforcing this procedure.

- Insuring subcontractors are aware of the requirements of this procedure.

## General Foreman

The General Foreman is responsible for;

- Insuring the job site is inspected per this procedure.

- Having any deficiencies that are found are corrected.

## Safety Specialist

- The Safety Specialist is responsible for;

Ensuring all job sites are being inspected daily and in accordance with this procedure.

Ensuring all affected personnel are trained in the proper inspection of job sites as prescribed in this procedure.

Conducting periodic inspections of job sites to insure inspections are being conducted properly.

Retaining copies of the daily inspection forms and reporting any deficiencies to the Department Manager.

## Employees

- Employees are responsible for;

Maintaining job sites in a safe manner as outlined by this procedure.

Reporting any unsafe condition to their foreman immediately or correcting the hazard if possible.

Using engineering controls, procedures, and/or ppe needed to work as safe as possible.

Affixing a Danger Do Not Use tag on any defective tools or equipment being stored on their job site.

Safe operation and condition of the tools and equipment that he/she uses.

# Housekeeping

One of the leading causes of work place injuries is housekeeping. The accumulation of trash, dirt, dust, improper tool and equipment storage can lead to slip, trip, and fall injuries; fires, flying debris, etc. Housekeeping inspections should include the following at a minimum;

Make sure the job site is free of excess trash and debris.

  - Are there enough trash receptacles.

  - Is ample time being allotted for debris cleanup and disposal?

Walkways and passage ways must be clear of ladders, tools, equipment, etc. that could cause an unsafe condition when walking through the area.

  - Lighting must be adequate for safe movement.

  - Floor openings must be covered by material strong enough to hold normal loads and be secured in place with the words “DO NOT REMOVE” printed in a visible location on the cover.

  - All obstructions at a height of seven (7) feet or less protruding into the walkways and passage ways must have some form of identification alerting workers to their presence.

All equipment and materials must be properly stored to prevent accidental discharge, damage, accidental operation, or misplacement.

Electrical cords, hoses, welding leads, etc must be ran above 7 feet in the overhead or along the side of a walkway (tied to mid rail if handrails are present) to prevent tripping hazards.

  - Cords, hoses and leads ran on the ground should not have loops, knots, or kinks in the section that is crossing the walkways.

Water kegs, trash receptacles, and restrooms must be in place and maintained for use.

  - Water kegs must have a secure top and have the words “DRINKING WATER ONLY” labeled on the outside of the keg. The lid must be secured with tape and the date it was filled printed on the tape.

  - A minimum of 1 toilet and urinal must be provided per 40 employees at a minimum.

Any hazard that cannot be eliminated must be identified by a protective or identifying means.

## General Waste Management

- Management will estimate the amount and types of wastes that will be generated at the job site prior to the start of the job so the need for containers and waste removal can be determined.

Scrap materials must be stored in a way that they do not impose a cut or puncher hazard to workers.

Wastes material will be stored and handled in ways to minimize the potential for a spill or impact to the environment.

During outdoor activities, receptacles must be covered to prevent dispersion of wastes materials and to control the potential for run-off.

All wastes will be segregated to ensure opportunities for reuse or recycling.

During the course of construction, alteration, or repairs, form and scrap lumber with protruding nails, and all other debris, shall be kept cleared from work areas, passageways, and stairs, in and around buildings or other structures.

Combustible scrap and debris shall be removed at regular intervals during the course of construction. Safe means shall be provided to facilitate such removal.

Containers shall be provided for the collection and separation of waste, trash, oily and used rags, and other refuse. Containers used for garbage and other oily, flammable, or hazardous wastes, such as caustics, acids, harmful dusts, etc. shall be equipped with covers. Garbage and other waste shall be disposed of at frequent and regular intervals.

# Personal Protective Equipment (PPE)

## Hard hats must be worn in the following manner;

  - Head band in the designed position (usually with size adjustor rests on the back of the head).

  - Hard hats must be free of cracks, discoloration, and melted spots.

  - Stickers on hard hats must inhibit an inspection of the integrity of it.

  - Must be class G at a minimum.

## Hearing protection

- Hearing protection must be worn in areas with noise levels above 85 decibels.

  - If you have to raise your voice in order to be heard you are in an area above 85 decibels.

## Safety Glasses

- Safety glasses at a minimum must be worn throughout the job site were hazards that could affect workers eyes are or could be present.

## Foot Protection

- Safety toed boots at a minimum are required to be worn at all job sites.

## Task Specific PPE

- Face Shields, goggles and other job specific PPE must be worn as potential hazards dictate.

# Falls Protection

- Workers must use fall protection when working in areas with a fall potential of greater than 6 feet.

- All anchors must have a holding capacity of 5,000 lbs unless they are engineered for fall protection.

- If work is being conducted on a roof with a pitch of less than 16 degrees a perimeter guarding system can be installed at a minimum of 10 feet from the edge and used as a means of fall protection.

- Static lines and rat lines must be able to withstand a load of 5,400 pounds of force unless they are designed by a certified engineer at which time they must be capable of supporting 3,500 pounds of force.

- Fall protection equipment must not be used for anything other than personnel fall protection.

- All fall protection equipment must be stored per the requirements of the fall protection procedure and inspections must be conducted per manufacture recommendations.

# Confined Space

- All aspects of either WW Gays Confined Space Procedure or the customers must be followed. Whichever offers the greater protection for the entrant.

- The confined space permit must be completely filled out.

- The attendant for the confined space must be trained and know the potential hazards in the space and how to summon help in the event of an emergency.

- If the entrance to the space is vertical and greater than 4 feet the entrant must be attached to a personnel hoist anchored to a device capable of supporting a load of 5,000 pounds.

# Scaffolds and Ladders

## Scaffolds

- All scaffolds must be erected under the direction of a person certified as a competent person.

- Access ladders must extend 3 feet above the access to the working level.

- Scaffolding must be inspected daily for damage and unauthorized alterations by a competent person prior to use.

## Ladders

  - Portable ladders must be the proper type for the job.

  - A step ladder cannot be used as an extension ladder.

  - Extension ladders must be set at a 45 degree angle and extend 3 feet past the working level it is being used to access.

- Portable ladders must be inspected for cracks, bent rungs, loose rivets, etc. prior to use.

# Hoisting & Lifting Equipment

- Chain falls, Come-a-longs, and all other lifting equipment must be free of defects such as;

      - Bent or stretched hooks.

      - Missing or broken keepers.

      - Stretched chain links.

      - Cracked housings.

- All hoisting and lifting equipment must have a current inspection (Must be tested every 6 months) and tagged.

- Load rating on hooks and housing of devices must be legible.

- Any rigging used on structures with sharp edges such as I-beams must be used with a softener.

- Employees rigging lifts must be trained in proper lifting techniques.

- All lifting procedures must be followed.

# Vehicles and Mobile Equipment

- All lights, horns, and alarms must be tested daily, and any system found not operating properly must be repaired prior to use.

- If a vehicle or mobile equipment is equipped with a seat belt it must be used when operating the equipment.

- Equipment fluid levels must be checked and maintained at proper levels daily.

- Equipment must only be used for the task it is designed to do (Example: An aerial lift platform cannot be used to lift equipment).

- Equipment must be kept at appropriate distances from overhead power lines.

- Employees operating equipment must have current training and certification to operate the machinery.

# Tools and Equipment

## Electrical Cords

- All electrical cords must be free of damage.

- Tape cannot be used for repairing damaged insulation.

- All electrical cords are current on inspection.

## Powered Hand Tools

Electric power tools shall either be of the approved doubled-insulated type or grounded according to National Electric Code (NEC) 250-114.

All guards that are part of each power operated tool shall be in place.

All Handles that are part of each power operated tool shall be in place.

Plug and cord-connected power tools shall be inspected for the following as applicable:

  - Missing, corroded, or damaged plug prongs

  - Frayed, worn, burned, or missing insulation

  - Exposed conductors

  - Loose or poor connections

  - Damaged or cracked cases

  - Burns or scorch marks

All power tools shall be equipped with a constant pressure switch or control and may NOT have a lock on control.

## Gas, Propane, & Hydraulic Powered Tools

All hand-held, gasoline-powered tools shall be equipped with a constant pressure throttle that will deactivate the power to the tool motion when the pressure is released.

All liquid and/or propane-powered tools hose fittings in good condition.

Hydraulic power tools have approved fire-resistant fluids in them.

All caps and plugs in place and in proper condition.

All hose connections properly made and secured.

All guards in place.

Case is free of damage and cracks.

## Attachments

- Insure all blades, bits, disks, chains and belts are sharp, tight, undamaged and ready for use.

## Hand Tools

- All hand tools must be free of broken tips, teeth, handles, or any other part.

- All tools such as cold chisels and center punches must not have any mushrooming metal or chipped ends.

- Handles on tools such as shovels, sledge hammers, booms, etc. must be free of splits, or cracks and must be maintained as they were manufactured (No cut down handles).

- Tool handles must be free of grease or oils that could cause a slip hazard when in use.

- All tools must be in the same form they were manufactured.

- Tools must be free of bends and signs of over stress.

# FIRE Protection

- All flammables must be stored in appropriate containers.

- Insure oxygen and combustibles are stored separately.

- All containers must be properly labeled.

- Fire extinguishers must be the appropriate size to handle the fire potential.

- Insure monthly inspection has been completed and tag signed.

- Insure annual inspection is completed and tag is affixed to the extinguisher.

- All hot work must be contained by the use of a screen, fire blanket, etc., if possible.

- Gauges and arrestors used on gas cylinders must be properly maintained and free of leaks.

# Excavations

- Excavation walls must be sloped 2:1 if the excavation is greater than five (5) feet deep.

- A means of egress must be provided every 25 feet.

- Spoil piles must be a minimum of 2 feet away from the edge of the excavation.

- All equipment must be a minimum of 4 feet away when people are in the excavation.

- The atmosphere of the excavation must be checked at the beginning of each workday and any time it is abandoned for more than 45 minutes.

- A barricade must be installed completely around the excavation.

- A competent person trained in excavations must inspect the excavation daily and make periodic inspections of the work being conducted in the excavation.

# Administrative

- All employees working on the job site must go through the customer required training and job specific training if required.

- Any employee supervising a WW Gay job must first go through the WW Gay supervisor safety training class.

- All subcontractors must meet all requirements of the customer prior to starting work.

- There must be at least one person trained in basic first aid and C.P.R. on the job site.

- A first aid kit properly stocked to handle potential hazards on job will be maintained at the job site and available to workers at all times.

- If job is lasting more than 1 week and WW Gay has an onsite office, an OSHA poster and 300 log will be posted in the office for the duration of the job.

- Updated list of approved worker compensation doctors available.

- First Report of Injury and Accident Investigation Report forms available.

- Supply of JSA, Confined Space, Daily Jobsite Inspection, Hot Work, Forms available.

- Copy of the WW Gay Safety Manual must be available onsite.

- First Report of Injury and Accident Investigation Report forms available.

- Extra safety glasses available.

- Ear plugs available.

- Face Shield and extra shields available.

- Burning goggles, welding hood and gloves available.

- Danger tape, Caution tape available.

- LOTO, Do Not Operate, and Barricade tags available.

# Procedures

- Weekly safety meetings must be attended by all workers on job site and documented on the weekly safety meeting form.

- All WW Gay procedures or customer procedures for work being conducted at job site must be followed. More stringent procedure will be followed.

- Pre-job briefing must be conducted daily per the WW Gay Pre-Job Briefing procedure and documented.

- All safety documents must be turned into the safety department weekly.

# Hazardous chemicals

- Insure MSDS book is available and has a MSDS for each chemical being used on job site.

- All chemical containers must have readable labels with chemical name and hazards to user listed on it.

- Insure all chemicals are being stored in their proper position.

- All chemical containers must have caps in place and secure.

- Any container chemicals are being stored in must be labeled “Chemicals Stored Inside”.

- Insure all chemical containers do not have chemical residue on outside of container.

# Completing the daily job assessment form

- Each section of the Daily Job Assessment Form will be reviewed and proper inspection of the listed equipment completed by either the General Foreman or their designee. It is recommended that after the initial inspection of the jab site the foreman conduct the inspection.

- The completed form will be turned into their department with his/her time sheet. The Inspection report will then be routed to the safety department for review and retention.

- Inspection records will be kept for a period of 3 months and discarded thereafter.

# Training

Employees shall be trained in the Proper inspection of a job site as well as the procedure for completing the Daily Job Assessment Form. The employee will also be trained on the process for turning in the competed form.

Employees will be trained on the proper disposal methods for wastes. This training will include the following:

- General instruction on disposal of non-hazardous wastes, trash, and scrap metals.

- General instruction on disposal of hazardous wastes if hazardous wastes are going to be generated.

Retraining shall be provided for each employee as necessary so that the employee maintains the understanding and knowledge acquired through the initial training.