| Job Description: |  |  |  |  |  |  |  |  |  | Date: |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Foreman: |  |  |  |  |  | Superintendant: |  |  |  |  |  |
| Housekeeping |  |  |  |  |  |  | Yes | No | NA | Comments |  |
|  | Scrap material free of sharp edges or other puncher hazards |  |  |  |  |  |  |  |  |  |  |
|  | Walkways and passageways clear |  |  |  |  |  |  |  |  |  |  |
|  | Materials and equipment properly stored |  |  |  |  |  |  |  |  |  |  |
|  | Cords, Hoses, welding leads, etc. ran in overhead or secured to handrail (no trip hazards) |  |  |  |  |  |  |  |  |  |  |
|  | Water Kegs, trash receptacles, and restrooms provided for work area |  |  |  |  |  |  |  |  |  |  |
|  | Barricades, warning signs, and other hazard notices posted |  |  |  |  |  |  |  |  |  |  |
|  | Other: |  |  |  |  |  |  |  |  |  |  |
| Personal Protective Equipment |  |  |  |  |  |  | Yes | No | NA | Comments |  |
|  | Hard hats worn and maintained as required |  |  |  |  |  |  |  |  |  |  |
|  | Hearing protection worn as required |  |  |  |  |  |  |  |  |  |  |
|  | Eye protection worn as required |  |  |  |  |  |  |  |  |  |  |
|  | Proper foot protection worn for job as required (safety toed boots at a minimum) |  |  |  |  |  |  |  |  |  |  |
|  | Face shield and/or goggles worn as required |  |  |  |  |  |  |  |  |  |  |
|  | Job specific PPE in use as required (Respirators, gloves, life lines, etc.) |  |  |  |  |  |  |  |  |  |  |
|  | Other: |  |  |  |  |  |  |  |  |  |  |
| Fall Protection |  |  |  |  |  |  | Yes | No | NA | Comments |  |
|  | Full body harness worn in proper manner |  |  |  |  |  |  |  |  |  |  |
|  | Retractable secured to a suitable anchor |  |  |  |  |  |  |  |  |  |  |
|  | Perimeter guarding in place |  |  |  |  |  |  |  |  |  |  |
|  | Static lines, rat lines installed and capable of supporting 5,000 lbs of force |  |  |  |  |  |  |  |  |  |  |
|  | Fall protection equipment used only for fall protection |  |  |  |  |  |  |  |  |  |  |
|  | Fall protection equipment maintained and has current inspection documented |  |  |  |  |  |  |  |  |  |  |
|  | Other: |  |  |  |  |  |  |  |  |  |  |
| Confined Space |  |  |  |  |  |  | Yes | No | NA | Comments |  |
|  | Customer required entry procedure being followed |  |  |  |  |  |  |  |  |  |  |
|  | Confined space entry permit completely filled out and at entry opening |  |  |  |  |  |  |  |  |  |  |
|  | Confined space has no hazardous atmosphere |  |  |  |  |  |  |  |  |  |  |
|  | Confined space attendant trained and posted at space |  |  |  |  |  |  |  |  |  |  |
|  | Entrants trained and notified of potential hazards inside the space |  |  |  |  |  |  |  |  |  |  |
|  | Tripod and personnel hoist onsite if vertical entry is 4 foot deep or greater |  |  |  |  |  |  |  |  |  |  |
|  | Customer notified of confined space entry |  |  |  |  |  |  |  |  |  |  |
|  | Rescue plan completed and communicated to entrants and attendant |  |  |  |  |  |  |  |  |  |  |
|  | Other: |  |  |  |  |  |  |  |  |  |  |
| Scaffolds & Ladders |  |  |  |  |  |  | Yes | No | NA | Comments |  |
|  | Built per specifications and under supervision of a competent person |  |  |  |  |  |  |  |  |  |  |
|  | Access and egress provided (ladder must extend 3 feet above working platform if possible) |  |  |  |  |  |  |  |  |  |  |
|  | Scaffold inspected daily and tagged correctly by a competent person |  |  |  |  |  |  |  |  |  |  |
|  | Proper ladder for the job being preformed and secured (Top and Bottom if possible) |  |  |  |  |  |  |  |  |  |  |
|  | Ladder at proper angle and extending past the landing by at least 3 feet |  |  |  |  |  |  |  |  |  |  |
|  | Ladder inspected and no defects found prior to use |  |  |  |  |  |  |  |  |  |  |
|  | Other: |  |  |  |  |  |  |  |  |  |  |
| Hoisting & Lifting Equipment |  |  |  |  |  |  | Yes | No | NA | Comments |  |
|  | Chain falls, come-a-longs, chokers, in good condition with current inspection tag affixed |  |  |  |  |  |  |  |  |  |  |
|  | Softeners used as required |  |  |  |  |  |  |  |  |  |  |
|  | Proper rigging techniques being used |  |  |  |  |  |  |  |  |  |  |
|  | Heavy lift procedure implemented if required |  |  |  |  |  |  |  |  |  |  |
|  | Other: |  |  |  |  |  |  |  |  |  |  |
| Vehicles / Mobile Equipment |  |  |  |  |  |  | Yes | No | NA | Comments |  |
|  | Lights, brakes, horns, and back up alarms working properly |  |  |  |  |  |  |  |  |  |  |
|  | Seat belts working if provided and being used |  |  |  |  |  |  |  |  |  |  |
|  | Properly maintained |  |  |  |  |  |  |  |  |  |  |
|  | Equipment is being used properly |  |  |  |  |  |  |  |  |  |  |
|  | Clearance from over head lines – 10’ LV & 20’ HV (High voltage is anything over 50,000 volts) |  |  |  |  |  |  |  |  |  |  |
|  | Operator has proper/current certification to operate equipment |  |  |  |  |  |  |  |  |  |  |
| Vehicles / Mobile Equipment – Con’t |  |  |  |  |  |  | Yes | No | NA | Comments |  |
|  | Equipment has been inspected and documented to be in proper working order |  |  |  |  |  |  |  |  |  |  |
|  | Carts and mules are being driven in a safe manner |  |  |  |  |  |  |  |  |  |  |
|  | Other: |  |  |  |  |  |  |  |  |  |  |
| Tools & Equipment |  |  |  |  |  |  | Yes | No | NA | Comments |  |
|  | Electrical cords in good condition and are current on inspection |  |  |  |  |  |  |  |  |  |  |
|  | All tools are in good condition and safe to use |  |  |  |  |  |  |  |  |  |  |
|  | All tools are being properly carried and stored |  |  |  |  |  |  |  |  |  |  |
|  | Pneumatic / hydraulic hose connections are properly secured |  |  |  |  |  |  |  |  |  |  |
|  | Proper tools are being used for the job being performed |  |  |  |  |  |  |  |  |  |  |
|  | GFCI’s are being used |  |  |  |  |  |  |  |  |  |  |
|  | All portable generators are grounded |  |  |  |  |  |  |  |  |  |  |
|  | Workers are properly trained in the use of powder actuated tools |  |  |  |  |  |  |  |  |  |  |
|  | Other: |  |  |  |  |  |  |  |  |  |  |
| Fire Protection |  |  |  |  |  |  | Yes | No | NA | Comments |  |
|  | All flammables being stored properly |  |  |  |  |  |  |  |  |  |  |
|  | Oxygen and combustibles being stored separately (50 feet apart or by a 2 hour fire wall) |  |  |  |  |  |  |  |  |  |  |
|  | Containers labeled as to content |  |  |  |  |  |  |  |  |  |  |
|  | Fire extinguishers properly located and correctly sized to handle the fire potential |  |  |  |  |  |  |  |  |  |  |
|  | Fire extinguisher properly inspected (monthly / annual) |  |  |  |  |  |  |  |  |  |  |
|  | Hot work containment in place such as screens and/or fire blanket for worker protection |  |  |  |  |  |  |  |  |  |  |
|  | Compressed gases cylinders, gauges and arrestors used and maintained properly |  |  |  |  |  |  |  |  |  |  |
|  | Other: |  |  |  |  |  |  |  |  |  |  |
| Excavations |  |  |  |  |  |  | Yes | No | NA | Comments |  |
|  | Excavation sloped and shored properly (>5’) |  |  |  |  |  |  |  |  |  |  |
|  | Access and egress provided every 25’ (>4’) |  |  |  |  |  |  |  |  |  |  |
|  | Spoil pile located a minimum of 2 feet away from edge of excavation |  |  |  |  |  |  |  |  |  |  |
|  | Atmosphere checked in trenches greater than 4 feet deep prior to entry |  |  |  |  |  |  |  |  |  |  |
|  | Barricades in place and sufficient |  |  |  |  |  |  |  |  |  |  |
|  | Competent person assigned to inspect trench daily and observe safety of workers in excavation |  |  |  |  |  |  |  |  |  |  |
|  | Other: |  |  |  |  |  |  |  |  |  |  |
| Administrative |  |  |  |  |  |  | Yes | No | NA | Comments |  |
|  | All workers have completed required training for the customer and the work they are doing |  |  |  |  |  |  |  |  |  |  |
|  | Supervisors have completed WW Gay supervisory training program |  |  |  |  |  |  |  |  |  |  |
|  | All workers have a current drug card (< 6 months old) |  |  |  |  |  |  |  |  |  |  |
|  | All sub-contractors are qualified to work on customers site |  |  |  |  |  |  |  |  |  |  |
|  | At least one person trained in CPR & 1st Aid on job site |  |  |  |  |  |  |  |  |  |  |
|  | First aid kit available and filled with proper supplies |  |  |  |  |  |  |  |  |  |  |
|  | OSHA poster and 300A log posted in job trailer |  |  |  |  |  |  |  |  |  |  |
|  | Other: |  |  |  |  |  |  |  |  |  |  |
| Procedures |  |  |  |  |  |  | Yes | No | NA | Comments |  |
|  | Weekly safety meeting documented |  |  |  |  |  |  |  |  |  |  |
|  | Procedures / permits being followed i.e., Confined Space, LOTO, Hot Work, etc. |  |  |  |  |  |  |  |  |  |  |
|  | Pre-Job Briefing / Job Hazard Analysis conducted and documented |  |  |  |  |  |  |  |  |  |  |
|  | Other: |  |  |  |  |  |  |  |  |  |  |
| Name of Person Conducting Audit: |  |  |  |  |  |  |  |  |  |  |  |
| Notes: |  |  |  |  |  |  |  |  |  |  |  |