Revision Record

| DATE | REVISION # | APPROVED BY | DESCRIIPTION |
| --- | --- | --- | --- |
| 9/17/2010 | 1 | Ed Lewis | Added section 4.4.1 Exiting Basket Above Ground section. |

All revisions must be approved by WW Gay Safety Officer

THIS PAGE INTENTIONALLY LEFT BLANK

TABLE OF CONTENTS

1.0	 PURPOSE AND SCOPE	5

2.0	 DEFINITIONS	5

3.0	 RESPONSIBILITIES	6

3.1	 Project Superintendent	6

3.2	 General Foreman	6

3.3	 Foreman	7

3.4 	Employees	7

4.0	 PROCEDURE	7

4.1	 General	7

4.2	 Pre-Start Inspection	8

4.3	 Before Operation	8

4.4 	During Operation	9

4.4.1	Exiting basket above ground	10

4.5	 Other Requirements	10

4.5.1	Reporting Problems or Malfunctions	10

4.5.2	Entanglement	10

4.5.3	Capacity Limitation	11

4.5.4	Fueling	11

4.5.5	Battery Charging	11

4.5.6	Platform Positioning	11

4.5.7	Miss-use as a Crane	12

4.5.6	Travel	12

4.6	 Shutdown of Aerial Platform	12

5.0	 TRAINING	12

THIS PAGE INTENTIONALLY LEFT BLANK

# 1.0		PURPOSE AND SCOPE

The purpose of this procedure is to outline the safe operating procedures of WW Gay owned and/or rented aerial platforms, articulating boom platforms, extensible boom platforms, man lifts, or a combination of such devices used to elevate personnel to work locations above ground and to prevent serious accidents from occurring while operating these devices.

This procedure applies to aerial lifts utilized by employees and sub-contractors at WW Gay.

# 2.0		DEFINITIONS

Aerial Platform - A manually propelled, or vehicle mounted device that has an adjustable position platform, supported from ground level by a structure or vehicle.

Base - The relevant contact points of the aerial platform that form the stability fulcrum (e.g., wheels, casters, outriggers, stabilizers, etc.)

Chassis - The integral part of the aerial platform that provides mobility and support for the elevating assembly.

Competent Person - An individual who, because of training and experience, is capable of identifying hazardous or dangerous conditions in powered platform installations and of training associates to identify such conditions.

Configuration - All positions in which an aerial platform or any part thereof can be placed within its intended operating limits.

Elevating Assembly - The mechanisms used to position the platform relative to the aerial platform chassis.

Guardrail System - A vertical barrier intended to prevent associates from falling to lower levels.

Hazardous Location - Any location that contains, or has the potential to contain, an explosive or flammable atmosphere as defined in ANSI/NFPA 505.

Instability - the quality or state of being unstable, likely to tip over.

Insulated Platform - A platform designed and tested to meet the specific electrical insulation ratings consistent with the manufacturer's identification plate.

Interlock - A control or mechanism that, under specific conditions, automatically allows or prevents the operation of another control or mechanism.

Lanyard - a flexible line or rope, wire rope, or strap which is used to secure the body belt or body harness to a deceleration device, lifeline or anchorage.

Modification/Modified - to make a change(s), temporary or permanent, to an aerial platform that affects the operation, stability, safety factors, rated load or safety of the aerial platform in any way.

Operator - A qualified person who controls the movement of the aerial platform.

Outriggers - Devices that increase the stability of the aerial platform and that are capable of lifting and leveling the aerial platform.

Platform - the portion of the aerial platform intended to be occupied by associates with their necessary tools and materials.

Platform Height - The vertical distance measured from the floor of the platform to the surface upon which the machine is supported.

Qualified Person - An associate who by reason of knowledge, experience, and training is certified and familiar with the operation to be performed and the hazards involved.

Rated Work Load - the designed carrying capacity of the aerial platform as specified by the manufacturer.

Stability - The quality, state of being stable, firmly anchored, not likely to tip over.

Stabilizers - Devices that increase the stability of the aerial platform but are not capable of lifting or leveling the aerial platform.

# 3.0		RESPONSIBILITIES

## 3.1		Project Superintendent

The Project Superintendent is responsible for implementing and enforcing this procedure.

## 3.2		General Foreman

The General Foreman is responsible for;

- Monitoring compliance with this procedure.

- Ensuring aerial lifts owned or rented by the facility meet the requirements of ANSI/SIA A92.5-1992, and are in good working order.

- Ensuring operating and maintenance manuals, as well as the inspection log, are present on all owned and rented aerial lifts utilized at the job site/facility.

- Ensuring all employees are trained and certified in the operation of aerial lifts of the type being utilized prior to their operating such equipment.

- Maintaining a current list of job site/facility employees trained and authorized to use aerial lifts.

## 3.3		Foreman

Foreman are responsible to ensure aerial are inspected in accordance with this procedure prior to use

## 3.4 	Employees

Employees are responsible for operating and using aerial platforms in accordance with the requirements of this procedure.

# 4.0		PROCEDURE

## 4.1		General

Only trained and authorized personnel shall operate an aerial platform/lift, extensible boom platform, aerial ladders, articulating boom platforms, vertical towers, or any combination of such devices.

- Personnel shall be trained on this procedure and on the operating manual of the specific device which is to be operated or work performed from.

Note: Operating and maintenance manuals shall be obtained from the supplier of a rented aerial platform upon delivery.

A copy of the operating manual, maintenance manual, and the log of inspections shall be kept with each aerial platform. These documents are considered an integral part of the aerial platform and are vital to communicate necessary safety information to users and operators. Rented and/or sub-contractor supplied lifts shall not be permitted on the job site without these documents.

NO aerial platform shall be modified or altered without the modifications or alterations being approved and certified in writing by the manufacturer. Records of all approved modifications and alterations, including written authorization from the manufacturer for the modification or alteration, shall be kept with the aerial platform as part of the operating and maintenance manual. The altering or disabling of interlocks or other safety devices is prohibited.

All MANUFACTURER'S SAFETY BULLETINS shall be complied with as received from the manufacturer, distributor, or rental equipment dealer and copies of them shall be kept with the aerial platform as part of the operating and maintenance manuals.

## 4.2		Pre-Start Inspection

Before use each day, a qualified person shall visually inspect the aerial platform and conduct a functional test in accordance with the manufacturer’s recommendations. (As dictated in the Operators Manual maintained with the lift) This inspection shall include as a minimum;

- Operating and emergency controls.

- Safety devices.

- Personal protective devices including fall protection.

- Air, hydraulic, and fuel system leaks.

- Cables and wiring harness.

- Loose and missing parts.

- Tires and wheels.

- Placards, warnings, and control markings.

- Outriggers, stabilizers, and other structures.

- Guardrail system.

- Items specified by the manufacturer.

The inspection shall be documented on the inspection form for WW Gay owned equipment and on the inspection form received from the lift supplier for rented equipment. Such documentation shall be retained with rented lifts for as long as the lift remains on the job site.

Aerial platforms that are not in proper operating condition shall be removed from service until repaired. A warning tag stating "DO NOT USE" shall be attached to the control panel of the aerial platform.

## 4.3		Before Operation

Before authorizing an operator to operate an aerial platform the Foreman shall:

- Ensure that everyone who will be working on the aerial platform has been properly trained on this procedure, the operating manual of the particular type of aerial platform to be used, and has been authorized to operate the specific unit.

- Determine that the purpose for which the aerial platform is to be used is within the scope of the intended applications defined by the manufacturer.

- Provide approved fall protection devices and other safety gear for all associates who will be working on the platform.

- Check the area in which the aerial platform is to be used for possible hazards such as, but not limited to:

- Drop-offs or holes.

- Bumps or floor obstructions.

- Debris.

- Overhead obstructions and high voltage conductors.

- Hazardous locations.

- Inadequate surface and support to withstand all load forces imposed by the aerial platform in all operating configurations.

- Wind and weather conditions.

- Other possible unsafe conditions.

- Presence of unauthorized persons.

## 4.4 	During Operation

The aerial platform shall be operated in accordance with this procedure. The Operator shall ensure the following before each elevation of the platform:

- That the aerial platform is operated on a surface within the limits specified by the manufacturer.

- That the outriggers, stabilizers, extendable axles, or other stabilizing methods are used as required by the manufacturer.

- That guardrails are installed and access gates or openings are closed per manufacturer's instructions.

- Fall protection, consisting of a lanyard and body harness shall be utilized by all personnel operating within the platform of an articulating or extensible boom lift.

- That the load and its distribution on the platform and any platform extension are in accordance with the manufacturer's rated capacity for that specific configuration.

- That there is adequate clearance from overhead obstructions.

- If the aerial platform is not equipped with a backup alarm (or it cannot be heard above surrounding noise level) a spotter will be used to insure the safety of the movement.

- That the minimum safe approach distances (MSAD) to energized power lines and any part of the equipment are maintained.

      - 20 feet minimum for lines operating at 50kv or below

      - 10 feet plus 1 foot for every 10kv above 50kv for lines operating above 50kv.

- That all safety precautions defined in this procedure and the Operating and Maintenance Manual for the particular model of aerial platform being used are followed during the operation of the aerial platform.

- That all personnel maintain a firm footing, with both feet, on the platform floor while working thereon.

- The use of planks, ladders, or any other device on the aerial platform for achieving additional height or reach is prohibited.

- The work area shall be barricaded off with appropriate hazard warning tape.

### 4.4.1	Exiting basket above ground

If an employee needs to exit a man lift basket while it is extended above the ground the following procedure will be used:

- Tie off to a 5,000 lb anchorage on the structure you are accessing.

- Unhook from the aerial lift basket.

- Use secure footing to exit the basket being careful not to stand on control panel.

- When reentering the basket reverse the process described above.

This procedure should be used anytime workers feet are not on the bottom of the basket.

## 4.5		Other Requirements

### 4.5.1	Reporting Problems or Malfunctions

The operator shall immediately report any problems or malfunctions that become evident during operation of the aerial platform to their Foreman. Any problems or malfunctions that affect the safety or operation of the aerial platform shall be repaired prior to continued use of the aerial platform.

### 4.5.2	Entanglement

Care shall be taken to prevent rope, electric cords, hoses, etc., from becoming entangled in the aerial platform.

### 4.5.3	Capacity Limitation

Aerial platform rated capacities shall not be exceeded when loads are transferred to the platform at any height.

### 4.5.4	Fueling

Liquid fueled equipment shall have the engine shut down while fuel tanks are being filled. Fueling shall be done in a well-ventilated area free of flame, sparks, or other hazards that may cause fire or explosion.

When replacing propane fuel tanks, the following practices shall be adhered to;

- Every effort shall be made to change fuel tanks outside facility buildings.

  - If pre-operational inspection indicates a lack of fuel necessary to complete the desired task, the operator shall move the vehicle to the exterior and change the fuel tank.

- When changing a fuel tank, the vehicle shall remain running while the tank is turned off. The vehicle shall be allowed to stall to ensure maximum evacuation of propane from the fuel lines prior to the tank being disconnected.

- Care shall be taken to avoid contact with cold valve areas when changing tanks.  Leather gloves shall be worn if necessary.

- The fuel line and seal shall be visually inspected prior to attaching the new fuel tank.

- Empty and full fuel tanks shall be stored in their designated locations.

- If a significant quantity of propane is inadvertently released during tank changing, the operator shall ensure proper ventilation is accomplished and the area is checked with a combustible gas indicator for flammable accumulations. Particular attention shall be paid to low areas, including sumps, pits, etc..

### 4.5.5	Battery Charging

Batteries shall be charged in a well-ventilated area free of flame, sparks, or other hazards that may cause fire or explosion.

### 4.5.6	Platform Positioning

The aerial platform shall not be positioned against another object to steady the platform.

### 4.5.7	Miss-use as a Crane

The aerial platform shall not be used as a crane.

### 4.5.6	Travel

Under all travel conditions, the operator shall limit travel speed according to conditions of ground surface, congestion, visibility, slope, locations of personnel, and other factors causing hazards of collision or injury to personnel.

## 4.6		Shutdown of Aerial Platform

The operator shall cease operation of the aerial platform in case of any suspected malfunctions, any hazard, or potentially unsafe condition that may be encountered. The aerial platform and/or the work area shall then be inspected and any malfunction or problem shall be corrected before further operation of the platform.

# 5.0		TRAINING

All WW Gay personnel who are to perform the functions of inspectors and/or operators of aerial platforms shall be trained on this procedure and either on the same model of aerial platform or one having operating characteristics and controls consistent with the one to be used during actual job site operation.

Operator training shall include, but not be limited to the following before actually operating the aerial platform or performing inspection or maintenance on an aerial platform:

- The intended purpose and function of each of the controls.

- The manufacturers recommended inspection points and their minimum acceptable standards.

- The manufacturer's operating instructions, maintenance requirements and safety rules.

- Familiarity with all decals, warnings, and instructions displayed on the aerial platform.

- The hazards associated with aerial lift operation.

- Safe aerial lift set-up, including energized line clearances.

- Fall protection requirements specific to aerial lift operation.

- Actions to be taken should a system failure occur during lift operation.

The operator trainee shall operate the aerial platform in an area free of obstructions under the direction of the qualified person for a time sufficient to determine that the trainee displays proficiency in knowledge and actual operation of the aerial platform.

Written records of aerial platform training shall include the trainee’s name, date of training, trainer's name and affiliation, topics covered and the original written test or tests taken. These records shall be maintained for three years.

Re-training shall take place every three years, whenever a new model aerial platform is utilized, or when an employee demonstrates he or she is less than proficiency in the operation, inspection and/or maintenance of aerial platforms.