	[image: image2.emf]
	REEDY INDUSTRIES

Safety Management System
	Doc No:
	GNWSTE

	
	
	Initial Issue Date
	4-1-21

	
	
	Revision Date:
	Initial Version

	GENERAL WASTE MANAGEMENT
	Revision No.
	0

	
	Next Review Date:
	4-1-22

	Preparation:  Safety Mgr
	Authority:  President
	Issuing Dept:  Safety
	Page:
	Page 1 of 6
- 1 -Page 1 of 6



Purpose

The purpose of this waste management strategy was developed to provide guidance and requirements necessary for efficient, effective and compliant waste management during construction and operations.

Scope

This procedure applies to all REEDY employees.  When work is performed on a non-owned or operated site, the operator’s program shall take precedence, however, this document covers REEDY employees and contractors and shall be used on owned premises, or when an operator’s program doesn’t exist or is less stringent.

Procedure
The REEDY Safety Manager or other designated person in his or her absence is accountable for managing waste and disposition of wastes generated at the work site.

Waste Estimation

Project wastes, trash, and/or scrap materials will be taken into consideration before work begins.  REEDY must estimate the waste that will be generated prior to work being performed so that the need for containers and waste removal, if necessary, can be determined.  

Each site will utilize the following for planning of dumpster scheduling and total non-hazardous dry waste material.  These figures do not include neither recycling nor waste minimization efforts and reflect no use of an incinerator.  Dumpster figures are based on a 40 yard container and can be modified if another size is used by changing the table below.


[image: image1.emf]SAMPLE ONLY - SOLID WASTE

Number of Employees 10            25               35               50               100            

Total Estimated Square Feet of Waste (@ 0.675 cu ft per person daily)

Daily 7                  17                   24                   34                   68                  

Weekly 47               118                 165                 236                 473                

Monthly (4.33 wks) 205             511                 716                 1,023             2,046            

Annual 2,455          6,138             8,593             12,276           24,551          

Total Estimated Weight of Waste (@ 4lb per person daily) 40               100                 140                 200                 400                

Weekly 280             700                 980                 1,400             2,800            

Monthly (4.33 wks) 1,212          3,031             4,243             6,062             12,124          

Annual 14,549       36,372           50,921           72,744           145,488        

Number of Total Dumpster Fills 40 yard dumpster 7x8x22 = 1,232 square feet

Daily 0.0              0.0                  0.0                  0.0                  0.1                 

Weekly 0.0              0.1                  0.1                  0.2                  0.4                 

Monthly (4.33 wks) 0.2              0.4                  0.6                  0.8                  1.7                 

Annual 2.0              5.0                  7.0                  10.0                19.9               


REEDY must coordinate with the project site or owner to ensure proper disposal of wastes or scrap materials.  
REEDY must ensure the owner client is aware of whether wastes and scrap materials will be taken off site by REEDY or will be disposed of on the owner client’s site.
Waste Segregation

· Do not mix waste streams

· Only place waste in the designated container, satellite accumulation area (SAA), recyclable accumulation area (RAA), universal waste accumulation area (UWAA) or designated dumpster.

Recycling

Wastes should be recycled whenever practicable.  REEDY will encourage proper segregation of waste materials to ensure opportunities for reuse or recycling occurs at each work site.  The collection of recycled material will reduce the total load on the environment.  Bins of sufficient size must be lined with a plastic bag and clearly labeled for use.  Posters from REEDY will be posted throughout the work site to encourage recycling.  Collection bins will also be placed in administrative areas will follow the following color guiding:

· Blue - Paper

· Green - Aluminum cans

· Yellow - Plastic

Cardboard will be flattened, staples and excess shipping tape removed.  No cardboard shall be placed in the dumpster used for the landfill.  

Waste Handling Matrix

Each work site will develop a Waste Handling Matrix (sample shown) that will:

· Address safe practices related to the immediate storage and handling of waste, scrap or leftover material.

· The handling, organization and storage of waste and scrap materials to minimize potential impact to the environment.  Waste materials shall be properly stored and handled to minimize the potential for a spill or impact to the environment.  During outdoor activities receptacles must be covered to prevent dispersion of waste materials and to control the potential for runoff.
	Waste Stream
	Location
	Activity Generating Waste
	Hazardous/Non Hazardous
	Safe Storage Practice
	Disposal Method
	PPE or

Other Precautions

	Aerosol Can Contents
	Equipment Repair Shop
	Puncturing of aerosol cans
	Hazardous
	SAA is self-contained  in the equipment  repair shop
	Ship to assigned site for recycling or disposal
	Read warnings before use of unit.

	Aerosol Can Puncturing Unit Filter
	Equipment Repair Shop
	Filter Changes
	Hazardous
	Place in designated labeled container
	Ship to assigned site for recycling or disposal
	Change filter every 3 months

	Aerosol Cans 
	Various Locations
	Painting, lubricants, cleaning
	Non-Hazardous if aerosol can is punctured and drained
	Place punctured aerosol can in RAA storage drum
	Crush RAA storage drum and place in the scrap metal dumpster from client.
	See "Scrap Metal" for waste stream management

	Ash
	Smart Ash Unit
	Incineration of acceptable waste
	Non Hazardous
	Dispose of Immediately
	Place in the Burnable Waste Dumpster
	Gloves
Goggles

	Automotive and Heavy Equipment Parts-Used
	Equipment Repair Shop and Fab Shop
	Replacement
	Non-Hazardous
	Place in RAA
	Returned to vendors for recycling
	Starters, Alternators, Pumps, Transmissions

	Batteries (Alkaline)
	Various Locations
	Battery Failures
	Universal Waste
	Place in the UWAA
	"D" cell and below are acceptable  in the Non-Burnable Waste Dumpster
	Ship to designated site for recycling or disposal

	Batteries (Lead Acid)
	Equipment Repair Shop and Fab Shop
	Battery Failures
	Universal Waste
	No storage allowed. Containment boxes are labeled and available in the shops.
	Lead acid batteries are returned to the Vendor upon removal
	Ship to designated site for recycling

	Batteries (NiCad)
	Various Locations
	Battery Failures
	Universal Waste
	UWAA in the equipment repair shop.
	Ship to assigned site for recycling or disposal
	Cell phones, radios

	Butane Torch Bottle 
	Various Locations
	Mechanic activities
	Excluded Hazardous if recycled
	Place drained Butane Torch Bottles in RAA storage drum
	Crush RAA storage drum and place in the scrap metal dumpster 
	Prosolv Butane Bottle processor l

	Cardboard/Office Paper
	Parts Department & Offices
	Shipping Boxes & Office Activities
	Non-Hazardous
	Place in RAA
	Place on pallet in RAA and band for shipment to assigned site for recycling.
	

	Computers Discarded
	Parts Department & Offices
	Replacement
	Non-Hazardous
	Place in RAA
	Ship to assigned site for recycling or disposal
	

	Diesel Filters-Used
	Equipment Repair Shop and Fab Shop
	Filter Changes
	Non-Hazardous
	RAA for drained and crushed used filters
	Drain for 12 hrs., crush and incinerate in Smart Ash unit
	Place metal in recycle metal dumpster

	Diesel Rags
	Various Locations
	Mechanic activities
	Non-Hazardous
	Oily waste rag in clear bags w/yellow stripes.
	Incinerated in Smart Ash unit
	See "Ash" for management and disposal

	Drained Diesel
	Equipment Repair and Fab Shop
	Draining diesel fuel and filters
	Non-Hazardous when burned as off-Spec fuel
	Place in "used oil" tank in the equipment repair shop and fab shop.
	Burned for energy recovery in clean burn multi-oil heating system. 
	

	Empty Paint Cans
	Various Locations
	Painting activities 
	Non-Hazardous
	No storage allowed
	Ship to assigned site for recycling or disposal
	Paint cans must be RCRA empty.

	Fluorescent Light Ballast
	Various Locations
	Failure
	Non-Hazardous unless they contain PCB's or DEHP
	None
	Place in Non-Burnable Dumpster
	Ballast will say on the label if it contains PCB's

	Fluorescent Light Bulbs
	 Shops, Office Areas
	Bulb replacement
	Universal Waste
	Place bulbs in their original container in the  RAA in the shop area
	Ship to assigned site for recycling or disposal
	Label bulbs "Used Bulb" when put into RAA. 

	Glass
	Various Locations
	Replacement
	Non-Hazardous
	None
	Place in Non-Burnable Dumpster
	Ensure glass containers are empty.

	Glycol Rags
	Equipment Repair Shop and Fab Shop
	Fluid Changes 
	Non-Hazardous
	Oily waste rag WAA's lined w/clear bags w/yellow stripes.
	Incinerated in Smart Ash unit
	Minimize use of absorbent rags

	Glycol-Used
	Equipment Repair Shop and Fab Shop
	Fluid Changes 
	Non-Hazardous
	RAA - self-contained tank on recycling unit
	Recycled in glycol recycling unit
	Recycling unit stored in shop

	Grinding Wheels
	Equipment Repair Shop and Fab Shop
	Grinding activities
	Non-Hazardous 
	None
	Place in Non-Burnable Dumpster
	 

	Hoses & Belts
	Equipment Repair Shop and Fab Shop
	Replacement
	Non-Hazardous 
	Place in Non-Burnable Dumpster 
	Place in Non-Burnable Dumpster
	Drain all fluids from hoses

	Metal Shavings/Cuttings
	Equipment Repair Shop and Fab Shop
	Fabricating activities
	Excluded Hazardous if recycled
	Placed in recycle metal dumpster or metal only RAA's
	Ship to assigned site for recycling or disposal
	Ensure there are no free flowing cutting fluids present before disposal.

	Oil Filters-Used
	Equipment Repair Shop and Fab Shop
	Oil filter changes
	Excluded Hazardous 
	RAA for drained and crushed used filters
	Drain for 12 hrs., crush and incinerate in Smart Ash unit
	Place metal in recycle metal dumpster

	Oil-Used
	Equipment Repair Shop, Fab Shop, Service Trucks
	Draining oil and filters
	Excluded Hazardous if burned for energy recovery
	Receiving sumps are located in the Equipment Repair Shop and Fab Shop
	Burned for energy recovery in clean burn multi-oil heating system. 
	Keep lids on receiving sumps at all times. DO NOT PUT SOLVENTS INTO USED OIL

	Oily Waste (rags, absorbents)
	Various Locations
	Mechanic activities, equipment drips and leaks
	Non-Hazardous
	Oily waste rag WAA's lined w/clear bags w/yellow stripes.
	Incinerated in Smart Ash unit
	Collected daily. See "Ash" for management and disposal

	Paint Waste (rags, rollers, brushes, etc.)
	Various Locations
	Painting activities
	Determine on per occurrence basis.  Use MSDS or testing
	If hazardous, store in the assigned area.  If non-hazardous, no storage is required.
	If hazardous, ship to assigned site for disposal.  If non-hazardous, place in burnable waste dumpster.
	Need to review MSDS, do analytical test, or use generator knowledge to make waste determinations.

	Parts Cleaner Rags
	Equipment Repair Shop
	Cleaning parts
	Non-Hazardous
	Oily waste rag WAA's lined w/clear bags w/yellow stripes.
	Incinerated in Smart Ash unit
	See "Ash" for management and disposal

	Scrap Metal
	Various Locations
	Fabrication activities & house cleaning
	Excluded Hazardous if recycled
	Placed in recycle metal dumpster or metal only RAA's
	Ship to assigned site for recycling or disposal
	 Eye Protection

Gloves

	Sodium Vapor/   Metal Halide Light Bulbs
	Various Locations
	Bulb replacement
	Universal Waste
	Place bulbs in their original container in the RAA.
	Ship to assigned site for recycling or disposal
	Label bulbs "Used Bulb" when put into RAA.

	Tires
	Various Locations
	Replacement
	Non-Hazardous
	None
	Ship to assigned site for recycling or disposal
	

	Toner Cartridges
	Offices
	Copiers, printers, fax machines
	Non-Hazardous
	Placed in original container in RAA
	Ship to assigned site for recycling or disposal
	Verify toner is expended before disposal.

	Water Scrubber Filter & Absorbents
	Equipment Repair Shop and Fab Shop
	Filtering sump water in shops
	Non-Hazardous
	None
	Incinerated in Smart Ash unit
	See "Ash" for management and disposal

	Welding Rods
	Various Locations
	Welding activities
	Excluded Hazardous 
	Placed in recycle metal dumpster or metal only RAA's
	Ship to assigned site for recycling or disposal
	See "Scrap Metal" for waste stream management

	Wood Waste
	Various Locations
	Various activities and shipping pallets
	Non-Hazardous
	Store on the far back corner of the pad or in the dump truck box if available.
	Ship to assigned site for recycling or disposal
	Pallets are refurbished and recycled when possible


Storage Requirements

REEDY must ensure project related wastes are stored and maintained in an organized fashion to encourage proper disposal and minimize risks to employees.  Proper waste receptacles must be provided for trash and materials that may be reused or recycled during a project.
PPE

For each site waste management plan REEDY shall determine a PPE matrix that includes gloves, hand protection, eye and face protection and/or other necessary PPE.
Education and Training

Employees shall be instructed on managing waste generated at the work site and on the proper disposal method of wastes.  Examples include:
· Instruction on the proper handling, storage and disposal of wastes and depending on the waste generated at the site to also include general instruction on disposal of non-hazardous wastes, trash or scrap materials.  If wastes generated are classified as hazardous then employees shall be trained to ensure proper disposal and compliance with regulations.
· Minimization methods to reduce waste.
· Recycling methods and proper PPE to be utilized.
	
	Printed on: 17 March 2021
	REEDY



[image: image2.emf]_1329058650.xls
Sheet1

		SAMPLE ONLY - SOLID WASTE

		Number of Employees		10		25		35		50		100

		Total Estimated Square Feet of Waste (@ 0.675 cu ft per person daily)

		Daily		7		17		24		34		68

		Weekly		47		118		165		236		473

		Monthly (4.33 wks)		205		511		716		1,023		2,046

		Annual		2,455		6,138		8,593		12,276		24,551

		Total Estimated Weight of Waste (@ 4lb per person daily)		40		100		140		200		400

		Weekly		280		700		980		1,400		2,800

		Monthly (4.33 wks)		1,212		3,031		4,243		6,062		12,124

		Annual		14,549		36,372		50,921		72,744		145,488

		Number of Total Dumpster Fills 40 yard dumpster 7x8x22 = 1,232 square feet

		Daily		0.0		0.0		0.0		0.0		0.1

		Weekly		0.0		0.1		0.1		0.2		0.4

		Monthly (4.33 wks)		0.2		0.4		0.6		0.8		1.7

		Annual		2.0		5.0		7.0		10.0		19.9





Sheet2

		





Sheet3

		






